This utility is unsupported PML freeware and represents one way to represent single-line structural steel drawings in DRAFT.

This utility was tested for use with PDMS 11.6 & 12.0.

OVERVIEW

This utility is basically used in a two-step procedure:

- In DESIGN, a customized menu is utilized to add FITT elements to SCTNs.  Four FITTs per SCTN are created.  Each FITT owns a TMPL which consists of either one CYLI or one NBOX.  Each CYLI/NBOX element has the Level attribute set to 7 for unique representation within DESIGN to mimic the DRAFT representation.  Two CYLIs represent the single-line portion on each of the SCTN ends.  Two NBOXs represent the negative geometry to “cutback” or blank-out the portion of the SCTN that coincides with the CYLI lengths.  The remaining portion of the SCTN represents the structural symbol at the mid-point of the overall length and represents the actual width dimension and orientation.

Here is an example of what you see in DESIGN:

- In DRAFT, a customized DEPT is utilized which owns the representation library (RPLB) that includes the necessary representation rulesets (RRST), rules (RRUL) and styles (STYL) elements.  The ruleset /DRA/COM/REPR/STRU/PLAN_L7 needs to be set at each VIEW that utilizes this utility.

Here is an example of what you see in DRAFT (with annotation):

INSTALLATION INSTRUCTIONS:

- Exit all sessions of PDMS.

  - For 11.6, copy these files to the appropriate folders:

appuserdef.pmlobj	- Copy to C:\AVEVA\PDMS11.6.SPx\pmllib\design\objects

SCTNcb.pmlfrm	- Copy to C:\AVEVA\PDMS11.6.SPx\pmllib\design\forms

USERDEF		- Copy to C:\AVEVA\PDMS11.6.SPx\pdmsui\des\addins

  - For 12.0, copy these files to the appropriate folders:

appuserdef.pmlobj	- Copy to C:\AVEVA\Plant\ PDMS12.0.SPx\pmllib\design\objects

SCTNcb.pmlfrm	- Copy to C:\AVEVA\Plant\PDMS12.0.SPx\PMLLIB\design\forms

USERDEF		- Copy to C:\AVEVA\Plant\PDMS12.0.SPx\pdmsui\des\addins

Note:  For 12.0 only, delete the pml.index file from the C:\AVEVA\plant\PDMS12.0.SPx\pmllib folder prior to entering DESIGN.

- In DRAFT:

  - Login to appropriate project as SYSTEM user.

  - Navigate to WORL *

  - Drag and drop the SCTNcb-DEPT.txt file in to the Command Window to create a new DEPT /PROJ/STD-LIBRARIES.

  - Savework

Note:  The DEPT /Project_Libraries must exist because one of the new RRULs refers to a STYL in that DEPT.

USER INSTRUCTIONS:

- Enter DESIGN.  You should see a separate toolbar with one icon that looks like this:

Click the icon to open single form with two tabs: