Importing a Backing Sheet from AutoCad

Note:  You must have Draft administration privileges before you proceed.

1. You will be creating a BACK element, which will be owned by a sheet library (SHLB).  Either find a suitable owner, or create a new one, noting the name.  Ex. SHLB    /DRA/MAS/BACKS/COMPANY

(The SHLB can be owned by any LIBY,  but to see the results of this exercise you may need to change the PDMS Draft default backing sheet pointer to point to the  correct SHLB..... SETTINGS > USER DEFAULTS )

2. Start AutoCad and open the backing sheet that is to be imported into PDMS.

3. From the AutoCad R14 application menu bar, choose Tools > Customize Menus.  Browse for and then Open CADCENTRE\pdms11.2\autodraftR13source\pdmsback.mnu.  Click on the Load button on the Menu Customization window.  If you like, you may open pdmsdrft.mnu and pdmssymb.mnu as well.  This will allow total access to all AutoDraft facilities.  You may open a .mns (menu source file), a .mnc (compiled menu file), or a .mnu (menu template file).  Note for R13: If Tools > Menus is not an option, you may type in MENU at the AutoCad command line.

3a. You may load any single pdms*.mnu, or you can add all three options to the ACAD menu group.  To do this, click on the Menu Bar tab on the Menu Customization window.  Select each of the PDMS Menu Groups in turn and insert the PDMS Frame, PDMS Draw, and PDMS Symbol menus before the Help menu, say.

4. From the AutoCad application menu bar, choose Tools > Load Application.  The Load AutoLISP, ADS, and ARX Files window will appear.  Click on the File... button.  Navigate to CADCENTRE\pdms11.2\autodraftR13 then open each .lsp file.  After all files are open, then select them all and click Load.

5. From the AutoCad command line type EXPLODE.  Window around the backing sheet and then Enter or right-click.  Alternatively, you may type EXPLODE ALL.  Note:  This procedure may not be necessary.

6. From the AutoCad application menu bar, choose PDMS Frame > Write sheet.  If this menu pick is not available, click Tools > Menus and load the appropriate .mnu, .mnc, or .mns file from the \autodraftR13source directory.  This command then prompts the user for the information needed to create the PDMS macro file:

Select all items to be made

into a PDMS Backing/Overlay Sheet:

Select objects:			Window around the backing sheet and then Enter or right-click.

Inch/<MM>:			I <Enter>

Overlay/<Backing>:		<Enter> to accept the default

Draft Sheet Library Name <CE>:	DRA/MAS/BACKS/COMPANY	(owner of the BACK)

Backing Sheet Name:		 DRA/MAS/BACKS/COMPANY/MOORE

Draft Command Filename:	C:\TEMP\BACK.MAC   (Name the macro file)

The PDMS AutoCad lisp routine will then generate a macro file.  Note: If a full pathname of the file is not entered, it will default to the AutoCad working directory.

7. As a PDMS Draft Administration user, go to the AutoDRAFT application:  Draft > AutoDRAFT

8. Select Import > SheetFrame	When the Load AutoDRAFT Sheet Frame form is displayed, enter the Directory and the Filename of the macro file, and then OK the form.  PDMS will ask you to confirm that you are Ready to load AutoCAD generated Sheet Frame.  Note:  The macro may be executed from the command line if desired.  Ex.  $M/C:\TEMP\BACK.MAC

Go to AutoCad and Tools > Customize Menus and click the Menu Bar tab to remove the PDMS options if desired.

If the backing sheet does not appear correctly, Check out the LIMITS in Autocad to make sure they are set to the EXTENTS of the drawing.  This can also be done by editing the macro file directly using any text editor.  At the top of the file there is a X and Y value that should correspond to the sheet size.  Remember, too, that the backing sheet must be exactly the same size as the SHEE element in Draft.

# Importing a Symbol from AutoCad

1. Start AutoCad and display the symbol that is to be imported into PDMS.

2. From the AutoCad R14 application menu bar, choose Tools > Customize Menus.  Browse for and then Open CADCENTRE\pdms11.2\autodraftR13source\pdmsback.mnu.  Click on the Load button on the Menu Customization window.  If you like, you may open pdmsdrft.mnu and pdmssymb.mnu as well.  This will allow total access to all AutoDraft facilities.  You may open a .mns (menu source file), a .mnc (compiled menu file), or a .mnu (menu template file).  Note for R13: If Tools > Menus is not an option, you may type in MENU at the AutoCad command line.

3. You may load any single pdms*.mnu, or you can add all three options to the ACAD menu group.  To do this, click on the Menu Bar tab on the Menu Customization window.  Select each of the PDMS Menu Groups in turn and insert the PDMS Frame, PDMS Draw, and PDMS Symbol menus before the Help menu, say.

4. From the AutoCad application menu bar, choose Tools > Load Application.  The Load AutoLISP, ADS, and ARX Files window will appear.  Click on the File... button.  Navigate to CADCENTRE\pdms11.2\autodraftR13 then open each .lsp file.  After all files are open, then select them all and click Load.

5. KEY IN:  ATTDEF  (FOLLOW PROMPTS)

6. HIT ENTER AFTER ATTRIBUTE MODES

7. ATTRIBUTE TAG:  	enter  PDMS_NAME

8. ATTRIBUTE  PROMPT:  enter  ENTER PDMS SYMBOL NAME (for example)

9. DEFAULT ATTRIBUTE VALUE:  press Enter (or return)

10. Justify/Style/<Start point>: pick height and angle of where you want PDMS_NAME (STEP 8) to      appear.

11. Select PDMS Symbol > Make Symbol or KEY IN:  PDMS_MAKE _SYMBOL then follow the prompts.

12. INSERT BASE POINT:  Select origin of symbol

13. SELECT OBJECTS:  Window your symbol and text

14. SCALE FACTOR<1>:  PRESS ENTER

15. ROTATION ANGLE<0>: PRESS ENTER

16. ENTER NAME: (Key in name you want to appear in Members list in PDMS)

- From the AutoCAD menu bar, select PDMS Symbol>Write  Symbol then follow the prompts.

- Window entire symbol and text

- KEY IN: INCH

- DRAFT SYMBOL LIBRARY NAME: KEY IN: NAME OF “SYLB” in which the symbol will be stored.

- DRAFT COMMAND FILE NAME: Key in the name of macro file to be run in PDMS AUTODRAFT  (EX. ZZZ)

- Now go into PDMS DRAFT as a SYSTEM USER

- GO TO Draft>Administration and then Draft > AutoDRAFT

- Select Import  > Symbols

- Fill in path to  the location of the macro file (EX. C:\temp)

- Fill in name of macro (Step 22  EX. ZZZ)

- Pull out the replace all symbols button

- Click OK

- That’s it, you should have it.

- Navigate to the specified library (Step 21)and look for your symbol name (Step 17).

# Resources

PDMS 11.2 Installation Guide – Page 41 of 58

8 Installing AutoDRAFT ................................................................................... 8-1

8.1 Compatibility ...................................................................................... 8-1

8.2 The AutoDRAFT Software................................................................. 8-1

8.3 Configuring AutoDRAFT with AutoCAD 13..................................... 8-2

8.4 Installing and Configuring AutoCAD ............................................... 8-2

8.5 Getting Started with AutoCAD ......................................................... 8-3

8.6 Application Macros for use with PDMS DRAFT .............................. 8-3

PDMS AutoDRAFT User Guide

PDMS DRAFT Administrator Application User Guide

PDMS DRAFT User Guide