HOW IT WORKS

Each  flanged  component  has a   BLRFarray   which  points to a   BOLTSET .  The  boltset  contains information  for each  of  the  bolt  holes in  the flange.   When a  flanged  joint is encountered  during isometric production PDMS uses the information stored in the boltset to determine the amount of bolt length and bolt diameter required for each hole of each component.

The preferred bolt type for the component is also stored in the boltset.

Once  PDMS  has determined the bolt length for each of  the components, the bolt types are examined and the most  superior type is fixed for the joint.  This type is then  referenced in the bolt specification to determine the bolt required from the corresponding BOLTREF in the spec.

The  single bolt  (SBOL)  determined  from  the specification  is  then  referenced  to  determine what  (if any) additional length(s) needs to be added to the accumulated fitting lengths.

The total  accumulated  length is then checked against the  lengths held in the length table for the bolt and a final required length established according to the following rules :-

If there is an exact match with the calculated length then use that length.

If there is a  length within +1mm of  the calculated length then use that length

Otherwise use the next length above the accumulated length.

The item code to be shown on the isometric is then extracted from the corresponding UDA in the length table.

CATALOGUE REQUIREMENTS

###### BOLT SETS

Bolting information for a component is stored in the Bolt Set (BTSE).

Each hole is referenced by a Bolt Point (BLTP) which stores the following information:

NUMB      Bolt Hole Number

BDIA         Bolt Diameter    (In millimetres to 2 decimal places)

BTHK        Bolt Length through the component

BTYPE      Bolt Type

In order to simplify the input of data, the  BTSE s  have been categorised by flange standard/rating. This fixes the number of  holes and the bolt diameter for each size,  leaving only the bolt  thickness and bolt type as possible variables.

BTHK

The   BTHK utilises  parameters  already  use by the component in the catalogue.  This removes the   need to create separate  BTSE s  for different   facings or for a manufacturers specific requirements.    In this way  components  such as  relief valves, which  may  have  an  extra  throat thickness, do not require a separate BTSE .                                                                                                                     It does mean, however, that the parameter used for flange  thickness/bolt  must be controlled so that the total number of BTSE s is reduced.

Hence the following convention has been used:-

For single bore components:

Any of  PARAM  4 , 5 , 6 , or 7 is used for flange thickness/Bolt

For two bore components

Any 2 of PARAM  4 , 5, 6 , or 7 is used for the flange thickness/Bolt (one for each bore)

For three bore components

Any 3 of PARAM  4 , 5, 6 , or 7 is used for the flange thickness/Bolt (one for each bore)

For four bore components

PARAM 4 is used for the 1st  flange thickness/Bolt

PARAM 5 is used for the 2nd flange thickness/Bolt

PARAM 6 is used for the 3rd flange thickness/Bolt

PARAM 7 is used for the 4th flange thickness/Bolt

This method does however mean the information needs to be kept in four separate BTSE references (one for each thickness parameter).

BTYPE

This can be one of the following:-

BOLT   (default)

MACH

STUD

CAP

JACK

TAP

Generally, the  bolt type  is governed by the  piping specification  but the length calculation method is the same for all types. For this reason most BTSE s have the bolt type set to BOLT for each hole and the bolt specification default will determine whether a studbolt or hex head bolt is to be used.

For tapped holes, where a specific bolt type is required, the type is set to TAP  and a separate entry is made in the bolt specification to override the default.

In principal any hole with a  BTYPE  other than  BOLT  will cause PDMS to use that  other bolt.  Thus, any of the other types may be used for a component to cause an override to occur.

Since each hole is handled  independently it is  possible to mix BTYPE s within the set.

Eg  Where a Butterfly valve requires through bolts for some holes and hex head screws for the rest.

(NOTE: In special cases the use of a “dummy” component may be considered to trigger a specific bolt selection)

NUMBERING FORMAT

Where  a choice  is restricted  by the system  the digits  chosen have  been fixed.

Where  there  is  no restriction  digits have  been used  in  alphabetical order.  Any additional entries should use the next available letter. This is much easier to control than trying to make things ‘meaningful’.

BTSE

Digits 1 & 2        BS    Bolt Set

Digit   3                Bolt Type                                                        Spec BTYP

B      Bolt  (default)                                                BOLT

C      Cap Screw                                                     CAP

L      Tapped Hole (Wafer Lug Valve)                 TAP

M     Machine Screw                                              MACH

S       Studbolt                                                         STUD

T       Tapped Hole                                                 TAP

Digit   4                Flange Standard

A      ANSI  B16.5

B      MSS SP44

C      API

D      SAE

Digit   5                Rating

From Standard PDMS Coding

Digit   6                Parameter Number

4, 5, 6, or  7

Digit   7                Nominal Bore

From Standard PDMS Coding

BLTP

###### The BLTP uses the number of the BTSE with a suffix –PTn  where n is the hole number.

###### (holes are numbered clockwise around the flange starting at the top)

###### SINGLE BOLT

The  single  bolt   (SBOL)   defines  the  additional  items  (nuts,  washers  and  extra thread),  used in conjunction with the flange dimensions to calculate the required bolt length plus reference to a length table.  Since  washers are not normally used, and  the nut and  extra thread  are consistent,  it is possible to define the SBOL in terms of its type, dimensional standard, material, and diameter.                            SBOL s are members of a BLIS

BLIS

Digits   1 & 2       BB   Bolt

Digit   3                Length Standard (for access to client length tables)

A     Client A

W    Whessoe

Digit   4                Length Units

I     Inch Lengths

M    Millimetre Lengths

Digit   5                Bolt Type

C      Cap Screw

H      Hex Head Bolt + Nut

J       Jack Screw

S       Studbolt

T       Hex Head Bolt - no nut – (Tapped Hole)

Digit   6                Bolt Material Standard

BOLT

A       ASTM A193-B7

B       ASTM A320-B8

C       ASTM A320-L7

D       ASTM F468-5

Digit   6                Nut Material Standard

A       ASTM A193-B8

B       ASTM A194-2H

C       ASTM A194-4

D       ASTM A194-7

E       ASTM A467-5

NOTE:- Where no nut required use 0

SBOL

Numbering of the SBOL is identical to the BLIS with an additional digit for bolt diameter.

Digit   7                Diameter

See Table

LENGTH TABLE

The length table (LTAB) defines the preferred lengths of bolt for each diameter and the stock code

to be printed on the isometric Bill of Materials and  MTO  reports.  The numbering format is

identical to that for SBOL except that the 2nd digit is L instead of B .

LTAB

Digits   1  & 2      BL   Length Table

Digit   3                length standard (for access to client length tables)

A     Client A

Digit   4                Length Units

I     Inch Lengths

M    Millimetre Lengths

Digit   5                Bolt Type

C      Cap Screw

H      Hex Head Bolt

S       Studbolt

T       Hex Head Bolt - no nut – (Tapped Hole)

Digit   6                Bolt Material Standard

BOLT

A       ASTM A193-B7

B       ASTM A320-B8

C       ASTM A320-L7

D       ASTM F468-5

Digit   6                Nut Material Standard

A       ASTM A193-B8

B       ASTM A194-2H

C       ASTM A194-4

D       ASTM A194-7

E       ASTM A467-5

NOTE:- Where no nut required use 0

DTAB

Numbering of the DTAB is identical to the LTAB with an additional digit for bolt diameter.

Digit   7                Diameter

See Diameter Table

DIAMETER TABLE

The following table has been created so that the same BOLTREF s and SBOL s will give the correct diameter of bolt whether Imperial or Metric diameters are required on output. In either case the PDMS input for diameter is in metric but will convert correctly on output.

The diameter  must  be input in the same format  (i.e. 2 decimal places) on all PDMS forms.

|  |  | CODE | METRIC | IMPERIAL | PDMS INPUT |
| --- | --- | --- | --- | --- | --- |
|  |  | A | M8 | - | 8.00 |
|  |  | B | M10 | - | 10.00 |
|  |  | C | M12 | - | 12.00 |
|  |  | D | M14 | 1/2 | 12.70 |
|  |  | E | M16 | 5/8 | 15.88 |
|  |  | F | M18 | - | 18.00 |
|  |  | G | M20 | 3/4 | 19.05 |
|  |  | H | M22 | - | 22.00 |
|  |  | I | M24 | 7/8 | 22.23 |
|  |  | J | M27 | 1 | 25.40 |
|  |  | K | M30 | 1.1/8 | 28.58 |
|  |  | L | M33 | 1.1/4 | 31.75 |
|  |  | M | M36 | 1.3/8 | 34.93 |
|  |  | N | M39 | 1.1/2 | 38.10 |
|  |  | O | M42 | 1.5/8 | 41.28 |
|  |  | P | M45 | 1.3/4 | 44.45 |
|  |  | Q | M48 | 1.7/8 | 47.63 |
|  |  | R | M52 | 2 | 50.80 |
|  |  | S | M56 | 2.1/4 | 57.15 |
|  |  | T | M60 | - | 60.00 |
|  |  | U | M64 | 2.1/2 | 63.50 |
|  |  | V | M68 | - | 68.00 |
|  |  | W | M70 | 2.3/4 | 69.85 |
|  |  | X | M72x6 | 3 | 76.20 |
|  |  | Y | M76x6 | 3.1/8 | 79.38 |
|  |  | Z | M80x6 | 3.1/2 | 88.90 |
|  |  | 1 | M90 | - | 90.00 |

SPECIFICATIONS

BOLT SPECIFICATION

The bolt specification is similar to the piping specification but is used differently by PDMS.

The headings are of the same format but with BDIA and BTYP replacing PBOR and STYP

HEADING

TYPE     NAME       BDIA      BTYP        CATREF     DETAIL    MATXT      CMPREF     BLTREF

TYPE        will always be BOLT

NAME      There is no definite established naming convention except for the spec name

prefix but it is probably reasonable to use the SBOL name wherever possible.

BDIA        always in mm to 2 decimal places

BTYP        The default (preferred) choice use BOLT  Use CAP, TAP,…etc for alternatives

CATREF  Although not actually used by PDMS an entry must be made here. There are

“dummy” components in the catalogue for this purpose.

Eg     /ISBOLTA  /IMBOLTA

DETAIL    Bolt description to appear on reports

CMPREF  Bolts do not generally have a CMPREF so use =0

BLTREF   This is the SBOL for the required bolt and is therefore the most important entry.

It is from this that the final bolt will be derived.

###### PIPING SPECIFICATION

In order to use the new bolting method and access the correct bolt spec. the piping specification must have two attributes set.

Bspec                must be set to the name of the bolt specification

Bltmethod         must be set to ‘NEW’

These attributes must be set in PARAGON

Bolt Diameter and Length Units to be used on Isometrics are set in ISODRAFT

IMPLEMENTATION

Assumption   A new piping specification has been received which includes flanged components to a standard/rating which has not previously been used. The Bolting for this specification is also new to the system.

A   BTSE

- Create a new CATA for the Flange standard/rating required. This name can be descriptive.

- Create a new SECT for the standard/rating/BTYP.  This name may also be descriptive.

- Create a new BTSE for the first nominal size assuming Parameter 4 will be used for the bolt

thickness when the component is made.

- Create a BLTP for each hole in order round the flange. Set the NUMB BDIA BTYP & BTHK

Eg

NUMB 1   BDIA 12.70  BTYP BOLT  BTHK PARAM 4

- Repeat for all sizes of flange.

- Repeat from 2  for Parameters 5  6 & 7

B   LTAB

- Create a new length table for the new bolt.

- Create new DTAB for the first diameter of bolt

2.1 Set acceptable blengths

BLEN ( 105mm 110mm 120mm )

  - Set stock codes  (example using :stocka)

:Stocka1 ‘  xxx.xxx   yyy.yyy  zzz.zzz  ’

Any number of lengths may be assigned to BLEN but the stock numbers are restricted by the number of characters and generally only ten entries can be made on one line of input. Further codes may be entered on separate lines by entering :Stocka2  and Stocka3 on subsequent lines. If this needs to be used then the stock codes must be sequential in the same order as the lengths.

Note:  The parenthesis () and quotes ‘’ are compulsory.

C   SBOL

- Create a BLIS for the new bolt.

- Create a SBOL for each bolt diameter.

  - Set the length table reference

Nstd  (name of DTAB)

  - Set the extra length  (This is the length of bolt to protrude beyond the nut [2 threads])

Xtra   10

  - Set the length of any additional items (nut washers etc) This is generally only the nut for normal bolting, 2 nuts if tensioning is required, 0 if no nut required.

Bitlen    38.5

D   COMPONENT

Create or edit the components to include the correct BLTREF

E    BOLT SPECIFICATION

Create the bolt specification so that for each diameter of bolt the correct SBOL is set as the BLTREF for each type of bolt required.