# InTouch Tag Server on Terminal Server

This InTouch application is a demonstration of a Tag Server application running on a Terminal Server.

A common requirement of customers using InTouch for Terminal Services is to have multiple operators running the same InTouch application.  The nature of Terminal Server is that each session is independent of the other sessions and also the Console.  This means that if each session opens the same window of the same InTouch application and changes the value of a slider linked to a memory tag, then each session will see a different value for the same memory tag.  In other words, each session has its own instance of the same memory tag.  Most Terminal Server applications do not want independence of memory tag values, but they need to have the memory tags centralized and also centralized alarms and history.  This centralization of memory tags is accomplished by using the Tag Server architecture.  Centralization of alarms and history is accomplished by configuration of the alarms and history so that only the Console application writes to history files and writes to the alarm database.  Note that no centralization of IO tags is necessary in the Intouch application, because the source of the IO tags are the IO Servers.

The Tag Server architecture is not new as it has been used in InTouch applications for years.  More information about this architecture can be found in the InTouch User’s Guide.  The implementation of Tag Server in a Terminal Server contains a  Tag Server application running on the Terminal Server Console and InTouch client applications running in the Terminal Server sessions that have remote tags referencing tags in the Tag Server application on the Console.

It is possible to have only one InTouch application that functions as both the Tag Server and as a client application.  The demo app included here is built to run on both the Console as the Tag Server and also on the sessions as a client application.

You must have InTouch 7.11 for Terminal Services or InTouch 8.0 for Terminal Services installed.

This demo can be run using InTouch 7.11 or InTouch 8.0.  It runs in both the Terminal Server Console as the Tag Server and also in the Terminal Server Sessions as a client application.  The demo contains alarms and history.  It is necessary to configure NAD for the demo to work.  With NAD each user has the application in a different folder and will not overwrite the history files in the master application on the Console.

To configure the demo follow these steps.

- In Alarm DB Logger Manager (outside of InTouch) configure the Alarm Query with the default query which is \InTouch!$System

- In the Application Startup script change davidu6 to the node name of the Console.

- In the Distibuted Alarm Object change davidu6 in the Alarm Query to the node name of the Console.

- In the Alarm DB View ActiveX Control in the tab Database configure Server Name, User and Password to connect to the alarm database.

- In Special/Configure/Distributed Name Manager/Distributed History change the UNC (Universal Naming Convention \\nodename\share\directory) to the directory that contains the master application (InTouch application on the Console).

- For each client PC follow the steps in “Configuring NAD for Terminal Services” on page 74 of the InTouch 8.0 manual “InTouch for Terminal Services Deployment Guide”.

- There is a problem (SR 2350240) with InTouch 7.11 Patch03 or earlier.  The NAD clients overwrite the history files on the Console.  The solution is to install InTouch 7.11 Patch04 or later.

- In the master application in Configure/Historical Logging :

  - Check “Enable Historical Logging”.

  - Change “Store log files in Specific Directory” to the folder that contains the application.  This could be a UNC reference.

  - Change “Name of Logging node” to the node name of the Console.

- For each client configure DRC (Dynamic Resolution Conversion) to avoid problems with different resolutions of the InTouch application between the clients and the Console.  For example, in Node Properties check “Allow WindowViewer to dynamically change resolution” and select “Convert to screen video resolution”.

- When running WindowViewere use the button Init Pen and later Set Pen to assign pen1.  It is necessary to assign the Pen to None when starting View from a Session.  You can click in the Hist Trend in View to see those pens that are not assigned – white field instead of “…unassigned…”